Computer Organization and Assembly Language
8086 Processor Study Notes
By Muhammad Mehrab Ali

+

Assembly Language

Assembly Language is a low-level programming language that communicates directly with the processor using mnemonics such as MOV, ADD, and SUB instead of binary machine code.

The System Bus is a communication pathway that connects the CPU, RAM, and Input/Output (I/O) devices, allowing them to exchange data, addresses, and control signals.

The CPU (Central Processing Unit) is the brain of the computer. It fetches, decodes, and executes instructions, performs arithmetic and logical operations, and controls the overall operation of the computer.

RAM (Random Access Memory) is a temporary (volatile) memory where programs and data are loaded while they are being executed by the CPU. Its contents are lost when the power is turned off.

# Cybersecurity Connection

## 1. Assembly Language

### Why is it important?

Assembly language lets you understand exactly what the CPU is doing ,unlike high-level languages (Python, Java, C++), assembly instructions are much closer to machine code.

### Where is it used?

- Reverse Engineering

- Malware Analysis

- Exploit Development

- Shellcode Writing

- Binary Analysis

- Firmware Analysis

- Debugging programs

### Example

A malware analyst might see instructions like:

MOV AX, 5

ADD AX, 3

CMP AX, 8

Without understanding assembly, it's difficult to determine what the malware is doing.

## 2. CPU (Central Processing Unit)

### Why should a cybersecurity professional understand the CPU?

Every program—including malware—runs on the CPU. Understanding how the CPU executes instructions helps you analyze software behavior.

### Used in:

- Reverse Engineering

- Malware Analysis

- Debugging

- Digital Forensics

- Exploit Development

When debugging, you'll often inspect registers such as AX, BX, CX, DX, and the Instruction Pointer (IP) to understand the program's execution.

## 3. RAM (Random Access Memory)

RAM is one of the most important areas in cybersecurity.

### Used in:

- Memory Forensics

- Malware Analysis

- Incident Response

- Digital Forensics

- Threat Hunting

### Why?

When malware runs, it is loaded into RAM. Even if the malicious file is later deleted, traces of its activity may still remain in memory while the system is running.

Security analysts examine RAM to find:

- Running malware

- Active processes

- Network connections

- Encryption keys

- Passwords (in some cases)

- Suspicious code

Popular memory forensics tools include:

- Volatility

- Rekall

## 4. System Bus

The system bus carries:

- Data

- Addresses

- Control signals

### Why does it matter?

Understanding the bus helps explain how information moves through a computer. This knowledge becomes useful when studying:

- Embedded Systems Security

- Hardware Security

- Firmware Security

- Computer Architecture

# 🛣️ How This Topic Fits into Your Cybersecurity

This topic is the foundation for many advanced areas:

Computer Organization

│

▼

Assembly Language

│

▼

Operating Systems

│

▼

Computer Memory

│

▼

Reverse Engineering

│

▼

Malware Analysis

│

▼

Exploit Development

│

▼

Advanced Offensive Security

Notice that everything starts with understanding how a computer works internally.

# 🎯 Interview Insight

Why is Assembly Language important in cybersecurity?

Assembly language helps me understand how software executes at the processor level. It is essential for reverse engineering, malware analysis, debugging, exploit development, and understanding memory-related vulnerabilities.

# 8086 MICROPROCESSOR

The 8086 is a 16-bit microprocessor developed by Intel. We study its architecture and Assembly Language because it helps us understand how a processor executes instructions and manages memory at a low level.

## Features of the 8086 Microprocessor

- It is a 16-bit microprocessor, which means it can process 16 bits of data at a time.

- It contains 16-bit registers for storing data and addresses.

- It has a 16-bit data bus, allowing it to transfer 16 bits of data between the CPU and memory in one operation.

- It has a 20-bit address bus, enabling it to generate 2²⁰ memory addresses.

- It can access up to 1 MB ( 2²⁰  ) (1,048,576 bytes) of memory.

## Inside the CPU

Before writing Assembly Language programs for the 8086 microprocessor, it is important to understand its internal organization, also known as the programming model.

The programming model includes the CPU's registers, flags, instruction pointer, and segment registers, which work together to execute instructions and access memory efficiently.

# REGISTERS OF THE 8086 MICROPROCESSOR

A register is a small, high-speed storage location inside the CPU. Registers store data, addresses, and instructions that the CPU needs while executing a program. Since registers are inside the CPU, they are much faster than RAM.

# 1. General Purpose Registers (Data Registers)

The 8086 has four 16-bit general-purpose registers. Each can also be divided into two 8-bit registers.

| Register | Full Name | Purpose |
| --- | --- | --- |
| AX | Accumulator Register | Used mainly for arithmetic, logical operations, multiplication, division, and input operations. It is divided into AH (High 8 bits) and AL (Low 8 bits). |
| BX | Base Register | Used to store data and memory addresses. It is commonly used as a base address when accessing memory. It is divided into BH and BL. |
| CX | Count Register | Used as a counter in loops, shifts, and string instructions. It is divided into CH and CL. |
| DX | Data Register | Used for output (O) operations. It is divided into DH and DL. |

Note: AX, BX, CX, and DX are all 16-bit registers. Each consists of a High (H) and Low (L) 8-bit part.

# 2. Segment Registers

The 8086 divides memory into four logical segments. Each segment register stores the starting address (segment address) of a memory segment.

Ram is divided into four segments. Stores the segment address of the block of RAM. From RAM data comes in segment register.

Ram has three types of  addresses : Offset , segment and physical ( which are store in these four segments)

| Register | Full Name | Purpose |
| --- | --- | --- |
| CS | Code Segment | Stores the starting address of the program instructions (code). |
| DS | Data Segment | Stores the starting address of variables and data. |
| SS | Stack Segment | Stores the starting address of the stack. |
| ES | Extra Segment | Stores the starting address of an additional data segment when needed. |

Memory Segments

- Code Segment (CS) → Program instructions

- Data Segment (DS) → Variables and data

- Stack Segment (SS) → Stack operations

- Extra Segment (ES) → Additional data

# 3. Pointer Registers

Pointer registers store offset addresses.

## SP (Stack Pointer)

- Points to the top of the stack.

- Always works with the Stack Segment (SS).

- Used during PUSH, POP, CALL, and RET instructions.

## BP (Base Pointer)

- Used to access data stored in the stack.

- Normally works with the Stack Segment (SS).

- Frequently used by procedures (subroutines) to access local variables and parameters.

# 4. Index Registers

## SI (Source Index)

- Stores the offset address of source data.

- Usually works with the Data Segment (DS).

- Mainly used in string operations.

## DI (Destination Index)

- Stores the offset address of destination data.

- Usually works with the Extra Segment (ES) during string operations.

- Used as the destination pointer.

# 5. Special Purpose Registers

## IP (Instruction Pointer)

- Stores the offset address of the next instruction to be executed.

- Always works together with the Code Segment (CS).

Complete instruction address = CS : IP

## FLAGS Register (Flag Register / Status Register)

The FLAGS Register, also called the Status Register, stores the current status of the CPU after arithmetic and logical operations.

It helps the processor decide whether to continue execution normally or perform conditional jumps.

Some important flags are:

- Carry Flag (CF)

- Zero Flag (ZF)

- Sign Flag (SF)

- Overflow Flag (OF)

- Parity Flag (PF)

# Ram has three types of addresses : Offset address , segment address and physical address

# Physical Address Calculation

The 8086 generates a 20-bit physical address as it has 20-bit address bus using the following formula:

Physical Address = (Segment Address × 10H) + Offset Address

## Example 1

Segment Address = 1230H

Offset Address = 0045H

Solution

1230H × 10H = 12300H

12300H + 0045H = 12345H

Therefore,

Physical Address = 12345H

## Example 2

Segment Address = FF2AH

Offset Address = 00BFH

Solution

FF2AH × 10H = FF2A0H

FF2A0H + 00BFH = FF35FH

Therefore,

Physical Address = FF35FH

LAB 1

EXERCISE 1.1

What is the register set on a 32-bitand 64-bit Intel machine?

SOLUTION:

## Register Set on a 32-bit Intel Machine (IA-32)

A 32-bit Intel machine (like older Pentium processors) has the following main registers:

### 1️)General Purpose Registers (32-bit)

- EAX – Accumulator

- EBX – Base register

- ECX – Counter register

- EDX – Data register

### 2️)Offset Registers

- ESP – Stack Pointer

- EBP – Base Pointer

- ESI – Source Index

- EDI – Destination Index

### 3)Segment Registers

- CS – Code Segment

- DS – Data Segment

- SS – Stack Segment

- ES, FS, GS – Extra segments

### 4)Special purpose Registers

- EFLAGS – Contains status flags (zero, carry, sign, overflow, etc.)

- EIP – Points to next instruction

## Register Set on a 64-bit Intel Machine (x86-64)

A 64-bit Intel machine (modern processors) extends the 32-bit registers and adds more.

### 1️)General Purpose Registers (64-bit)

- RAX

- RBX

- RCX

- RDX

- RSP

- RBP

- RSI

- RDI

### 2️)Additional Registers (New in 64-bit)

- R8

- R9

- R10

- R11

- R12

- R13

- R14

- R15

### 3️)Instruction Pointer

- RIP

### 4️)Segment Registers

- CS, DS, SS, ES, FS, GS
(Note: segmentation is mostly not used in 64-bit mode.)

### 5️)Flag Register

- RFLAGS

# Basic Structure of Every Assembly Program

Every Assembly Language program follows a basic structure. Each section has a specific purpose, and understanding these sections is essential before writing any Assembly program.

org 100h

.data

; Variables are declared here

.code

main proc

; Program instructions

mov ah,2h              OR       mov aah,9

int 21h                                int 21h

mov ax,4C00h

int 21h

main endp

end main

## ORG 100H

org 100h

ORG stands for Origin.

It is an assembler directive that tells the assembler where the program will start in memory. In EMU8086, COM programs always begin execution at memory location 100H (Hexadecimal) because the first 256 bytes are reserved by DOS for the Program Segment Prefix (PSP). Since ORG is a directive, it is not converted into machine code and is not executed by the CPU.

## .DATA

.data

The .DATA section is used to declare all variables, constants, strings, and messages that the program will use.

Example:

name db 'Muhammad Mehrab Ali','$'

In this example, a string named name is created. The DB ( define byte ) directive reserves memory for the characters, and the dollar sign ($) marks the end of the string so that DOS Function 09H knows where to stop printing.

## .CODE

.code

The .CODE section contains all executable instructions. Everything written inside this section is executed by the CPU in sequence.

Example:

mov ax,10

add ax,5

These instructions are written inside the .CODE section.

## MAIN PROC

main proc

PROC stands for Procedure.

A procedure is a block of code that performs a specific task. The statement main proc marks the beginning of the main procedure, where program execution starts.

## MAIN ENDP

main endp

ENDP stands for End Procedure.

It marks the end of the procedure that was started with PROC.

## END MAIN

end main

This statement tells the assembler that the source program has finished. It also specifies that execution begins from the main procedure.

# Registers

Registers are very small, high-speed storage locations inside the CPU. They are used to temporarily store data, addresses, and intermediate results while the program is running. Since registers are located inside the processor, they are much faster than RAM.

## General Purpose Registers

### AX (Accumulator Register)

The AX register is the most commonly used register in Assembly Language. It is mainly used for arithmetic operations, multiplication, division, input/output operations, and moving data.

Example:

mov ax,10

This instruction stores the value 10 inside the AX register.

AX is a 16-bit register and is divided into two 8-bit registers:

- AH = Higher 8 bits

- AL = Lower 8 bits

Both AH and AL together form the complete AX register.

### BX (Base Register)

The BX register is generally used for storing data and memory addresses. It can also be used in arithmetic operations.

Example:

mov bx,5

This instruction stores the value 5 inside BX.

### CX (Counter Register)

The CX register is called the Counter Register because it is mainly used with the LOOP instruction. It stores the number of times a loop should execute.

Example:

mov cx,10

This instruction sets the loop counter to 10.

loop again

Each time the LOOP instruction executes, CX decreases by 1. When CX becomes 0, the loop automatically stops.

### DX (Data Register)

The DX register is mainly used for input and output operations. The lower part of DX, called DL, is commonly used to store a character before displaying it on the screen.

Example:

mov dl,'A'

mov ah,2

int 21h

The character stored in DL is displayed on the screen.

# MOV Instruction

The MOV instruction means Move, but it actually copies data from the source operand to the destination operand. The original value in the source remains unchanged.

### Syntax

MOV Destination, Source

### Example 1

mov ax,10

This instruction copies the value 10 into the AX register.

### Example 2

mov bx,ax

This instruction copies the contents of AX into BX.

Important Rules of MOV

- Both operands must have the same size.

- Data can be copied from register to register.

- Data can be copied from memory to register.

- Data can be copied from register to memory.

- MOV only copies data; it does not delete the original value.

# DB Directive

DB stands for Define Byte.

It is used to reserve one or more bytes of memory and store characters, numbers, or strings.

Example:

number db 5

This stores the number 5 in memory.

Example:

letter db 'A'

This stores the character A.

Example:

name db 'Ali','$'

This stores the string Ali followed by the dollar sign ($).

# String

A string is a sequence of characters stored together in memory.

Example:

db 'Hello World','$'

When displaying a string using INT 21H Function 09H, the string must end with the dollar sign ($).

The $ character tells DOS where the string ends. If the dollar sign is missing, the computer may continue printing random characters from memory until it accidentally finds a $.

# OFFSET

The OFFSET operator returns the starting memory address of a variable or string.

Example:

mov dx,offset name

This instruction loads the address of the variable name into the DX register so that DOS knows where the string begins.

# @DATA

Before a program can use variables declared in the .DATA section, the Data Segment register (DS) must point to that section.

First, the address of the data segment is loaded into AX:

mov ax,@data

Then the value in AX is copied into DS:

mov ds,ax

After these two instructions, the program can correctly access all variables and strings declared inside the .DATA section.

# Interrupt 21H (INT 21H)

## What is an Interrupt?

An Interrupt is a signal that temporarily stops the execution of the current program and asks the Operating System (DOS) to perform a specific task. After completing that task, the Operating System returns control to the program.

In 8086 Assembly Language, INT 21H is the most commonly used interrupt because it provides many DOS services such as taking input from the keyboard, displaying characters, displaying strings, and terminating the program.

The general syntax is:

mov ah, 21h

int 21h

The value stored in the AH register determines which DOS service will be executed.

# INT 21H Function 01H (Keyboard Input)

Function 01H is used to read a single character from the keyboard.

### Syntax

mov ah,1h

int 21h

After execution:

- The character entered by the user is automatically stored in the AL register.

- The typed character is also displayed on the screen automatically.

### Example

mov ah,01h

int 21h

If the user presses A, then:

AL = 'A'

# INT 21H Function 02H (Display One Character)

Function 02H is used to display a single character on the screen.

Before calling the interrupt, the character must be stored in the DL register.

### Syntax

mov dl,'A'

mov ah,2h

int 21h

Output:

A

### Important Points

- Displays only one character.

- Character must be placed in DL.

- AH must contain 02H.

# INT 21H Function 09H (Display String)

Function 9H is used to display a complete string.

The starting address of the string must be stored in DX.

### Syntax

mov dx,offset message

mov ah,9h

int 21h

### Example

.data

message db 'Hello World','$'

mov ax,@data

mov ds,ax

mov dx,offset message

mov ah,09h

int 21h

Output

Hello World

### Important Rules

- Function 09H prints only strings.

- The string must end with $.

- The address of the string is passed through DX using OFFSET.

# INT 21H Function 4C00H (Terminate Program)

Function 4C00H ends the program and returns control to DOS.

### Syntax

mov ax,4C00h

int 21h

Every Assembly program should end with these two instructions.

# Difference Between AH = 01H, 02H and 09H

### AH = 01H

Purpose: Input one character from the keyboard.

Input Register:

AH = 01H

Output Register:

AL

------------------------

mov ah,1h

int 21h

After execution:

- The character entered by the user is automatically stored in the AL register.

- The typed character is also displayed on the screen automatically.

### AH = 02H

Purpose: Display one character and numbers.

mov ah,2h

int 21h

Character should be stored in:

DL

### AH = 09H

Purpose:

mov ah,9h

int 21h

Display a complete string.

Requirements:

- String address in DX.

- String ends with $.

### AH = 4CH

Purpose:

mov ax,4C00h

int 21h

Terminate the program.

# Character Input

To take one character as input:

mov ah,01h

int 21h

The entered character is stored in AL.

Example:

If the user types

M

Then

AL = 'M'

# Character Output

To display one character:

mov dl,'A'

mov ah,2h

int 21h

Output

A

# String Output

A string is displayed using Function 09H.

Example

.data

msg db 'Foundation University','$'

mov ax,@data

mov ds,ax

mov dx,offset msg

mov ah,09h

int 21h

Output

Foundation University

# Carriage Return (0DH)

ASCII code 0DH is called Carriage Return (CR).

It moves the cursor to the beginning of the current line.

Syntax

mov dl,0Dh

mov ah,2h

int 21h

# Line Feed (0AH)

ASCII code 0AH is called Line Feed (LF).

It moves the cursor down to the next line.

Syntax

mov dl,0Ah

mov ah,02h

int 21h

# New Line

To move the cursor to a completely new line, both instructions are used together.

mov dl,0Dh

mov ah,2h

int 21h

mov dl,0Ah

mov ah,02h

int 21h

# ASCII Code

ASCII stands for American Standard Code for Information Interchange.

Every character stored in the computer has a unique numeric value.

Examples

'A' = 65

'B' = 66

'C' = 67

'a' = 97

'b' = 98

'0' = 48

'1' = 49

# Why Do We Use ADD 30H?

### ADD 30H (Screen par sahi cheez DIKHANE ke liye)

Jab computer ke andar ek original number (jaise 5) betha hai aur aap use screen par Display karwana chahte hain, toh aap ADD 30H karte hain taake monitor use pehchan sake.

- Flow: Computer Number (5H)  , then , ADD 30H   , then , 5h is converted into ASCII Code (35H)   , then , Screen par '5' Nazar aaya.

The processor stores numbers in binary.

Suppose:

mov al,5

The value 5 is stored as a binary number, not as the character '5'.

If we display it directly using INT 21H Function 2H, an incorrect symbol appears.

To convert the number into its ASCII character, we add 30H.

Example

mov al,5

add al,30h

mov dl,al

mov ah,2h

int 21h

Output

5

# Why Do We Use SUB 30H?

### SUB 30H (Keyboard se lekar MATHS karne ke liye)

Yahan dhyan dein: Jab user keyboard se koi ascii value Enter karta hai, toh computer ke paas direct ASCII value (35H) aati hai.Agar aap is value ko bina change kiye direct display karwa dein (INT 21H Function 2H se), toh screen par wapas '5' hi nazar aayega! Kyunki input bhi ASCII tha aur display ko bhi ASCII chahiye. Wahan SUB 30H ki zaroorat nahi hai.

SUB 30H ki zaroorat tab parti hai jab aap ko us entered number ke sath koi calculation (Plus, Minus, Multiply) karni ho.

#### Chalein ek example dekhte hain:

Farz karein user ne keyboard se enter kiya '5' (computer ko mila 35H) aur phir enter kiya '2' (computer ko mila 32H). Ab aap chahte hain computer inko plus kare.

- Agar aap bina SUB kiye plus karenge:

35H + 32H = 67H (ASCII table mein 67H ka matlab character 'g' hota hai, jo ke galat output hai).

- Sahi tareeqa (SUB 30H ke sath):

  - First input (35H) se 30H minus kiya   , then , Ban gaya 05H (Original No.)                                                                      .

  - Second input (32H) se 30H minus kiya   , then , Ban gaya 02H (Original No.)                                                            .

  - Ab plus karein: 05H + 02H = 07H (Bilkul sahi math!)                                                                 .

  - Ab is 07H ko Display karne ke liye wapas ADD 30H karein   , then ,  Ban gaya 37H (Screen par '7' nazar aa jayega).

When the user enters a number from the keyboard, the computer stores its ASCII code.

Example

User enters

5

The computer stores

35H

To convert it back into the numeric value 5, subtract 30H.

Example

sub al,30h

Now AL contains the actual number.

# Important Difference Between Character and String

A character is only one symbol.

Example

A

A string is a collection of characters.

Example

Assembly Language

Character is displayed using:

AH = 02H

String is displayed using:

AH = 09H

# Important Viva Questions

Q1. Which interrupt is mostly used for input and output?

Answer:

INT 21H

Q2. Which register selects the DOS function?

Answer:

AH

Q3. Which register stores one character for output?

Answer:

DL

Q4. Which register stores the character entered by the user?

Answer:

AL

Q5. Which function displays a string?

Answer:

AH = 09H

Q6. Why is '$' used at the end of a string?

Answer:

Because DOS Function 09H continues printing until it encounters the $ character, which marks the end of the string.

Q7. Which function ends the Assembly program?

Answer:

AH = 4CH

# Arithmetic Instructions in Assembly Language

Arithmetic instructions are used to perform mathematical operations such as addition, subtraction, multiplication, division, increment, and decrement. These instructions work on registers or memory locations and store the result in a register.

# ADD Instruction (Addition)

The ADD instruction adds the source operand to the destination operand and stores the result in the destination.

## Syntax

ADD Destination, Source

## Example 1

mov al,5

mov bl,3

add al,bl

### Explanation

- AL = 5

- BL = 3

- ADD AL,BL means 5 + 3

- Result = 8

- AL now contains 8

## Example 2

mov ax,10

add ax,20

Result:

AX = 30

# SUB Instruction (Subtraction)

The SUB instruction subtracts the source operand from the destination operand.

## Syntax

SUB Destination, Source

## Example

mov ax,15

mov bx,5

sub ax,bx

### Explanation

15 − 5 = 10

After execution,

AX = 10

# INC Instruction (Increment)

The INC instruction increases a value by 1.

## Syntax

INC Register

## Example

mov al,9

inc al

Result:

AL = 10

Instead of writing

add al,1

we simply write

inc al

# DEC Instruction (Decrement)

The DEC instruction decreases a value by 1.

## Syntax

DEC Register

## Example

mov al,8

dec al

Result

AL = 7

Instead of writing

sub al,1

we write

dec al

# MUL Instruction (Multiplication)

The MUL instruction is used for multiplication.

Unlike ADD and SUB, the result is stored automatically in special registers.

## Syntax

MUL Register

## 8-bit Multiplication

mov al,5

mov bl,4

mul bl

### Explanation

5 × 4 = 20

Result:

AX = 20

The processor multiplies AL by BL, and the result is stored in AX.

## 16-bit Multiplication

mov ax,100

mov bx,2

mul bx

The processor multiplies AX by BX.

The result is stored in:

DX:AX

# DIV Instruction (Division)

The DIV instruction divides one number by another.

## Syntax

DIV Register

## Example

mov ax,20

mov bl,4

div bl

### Explanation

20 ÷ 4 = 5

Result

AL = Quotient = 5

AH = Remainder = 0

If

21 ÷ 4

then

AL = 5

AH = 1

because

21 = 5 × 4 + 1

# Comparison of Arithmetic Instructions

| Instruction | Purpose |
| --- | --- |
| ADD | Adds two numbers |
| SUB | Subtracts one number from another |
| INC | Increases a value by 1 |
| DEC | Decreases a value by 1 |
| MUL | Multiplies two numbers |
| DIV | Divides one number by another |

# LOOP Instruction

The LOOP instruction repeats a block of code several times.

It automatically uses the CX register as a counter.

## Syntax

mov cx,5

Again:

; Instructions

loop Again

### Working

Suppose

mov cx,5

The CPU executes the loop 5 times.

Each time:

- CX decreases by 1.

- When CX becomes 0, the loop automatically stops.

# Label

A Label is a name given to a location in the program.

Example

Again:

mov dl,'A'

mov ah,2

int 21h

loop Again

Here,

Again

is the label.

Whenever the LOOP instruction executes, the processor jumps back to the label.

# Uppercase and Lowercase Characters

ASCII codes:

A = 65

B = 66

Z = 90

a = 97

b = 98

z = 122

Difference between uppercase and lowercase letters:

32

# Convert Lowercase to Uppercase

Subtract 32 from the lowercase letter.

Example

mov ah,1h

int 21h

sub al,32

mov dl,al

mov ah,2h

int 21h

If the user enters

a

ASCII value:

97

97 − 32 = 65

65 is A

Output:

A

Code:-

org 100h

.code

main proc

mov ah,1h

int 21h

mov bl,al

mov dl,0dh

mov ah,2h

int 21h

mov dl,0ah

mov ah,2h

int 21h

sub bl,32

mov dl,bl

mov ah,2h

int 21h

main endp

end main

# Convert Uppercase to Lowercase

Add 32.

Example

add al,32

Example:

A = 65

65 + 32 = 97

97 = a

Code:-

org 100h

.code

main proc

mov ah,1h

int 21h

mov bl,al

mov dl,0dh

mov ah,2h

int 21h

mov dl,0ah

mov ah,2h

int 21h

add bl,32

mov dl,bl

mov ah,2h

int 21h

main endp

end main

# Display a Character Multiple Times

Use a loop.

Example

mov cx,10

Again:

mov dl,'*'

mov ah,2

int 21h

loop Again

Output

**********

# Display A to Z

Example

mov cx,26

mov bl,'A'

Again:

mov dl,bl

mov ah,2

int 21h

inc bl

loop Again

Output

ABCDEFGHIJKLMNOPQRSTUVWXYZ

# Program Termination

Every DOS Assembly program should end with:

mov ax,4C00h

int 21h

This returns control to the Operating System.

# Common Mistakes

### Forgetting '$' after a string

Wrong

msg db 'Hello'

Correct

msg db 'Hello','$'

### Forgetting to initialize DS

Wrong

mov dx,offset msg

Correct

mov ax,@data

mov ds,ax

mov dx,offset msg

### Using AH = 02H to print a string

Wrong

mov dx,offset msg

mov ah,2

AH = 02H displays only one character.

Correct

mov dx,offset msg

mov ah,9

### Forgetting to load the character into DL

Wrong

mov ah,2

int 21h

Correct

mov dl,'A'

mov ah,2

int 21h

# Important Viva Questions

Q1. Which register is used by the LOOP instruction?

Answer: CX Register.

Q2. Which instruction increases a value by one?

Answer: INC.

Q3. Which instruction decreases a value by one?

Answer: DEC.

Q4. Which instruction is used for multiplication?

Answer: MUL.

Q5. Which instruction is used for division?

Answer: DIV.

Q6. What is the difference between uppercase and lowercase ASCII letters?

Answer: 32.

Q7. Which instruction converts a lowercase letter into uppercase?

Answer: SUB AL,32.

Q8. Which instruction converts an uppercase letter into lowercase?

Answer: ADD AL,32.

Q9. Which register controls the number of loop iterations?

Answer: CX Register.

# Lab 2

# Activity 2.1 : Hello World Program

## Objective

The purpose of this program is to learn how to display a string on the screen using INT 21H Function 09H.

## Program

org 100h

.data

hello_message db 'Hello',' World',0dh,0ah,'$'

.code

main proc

mov ax,@data

mov ds,ax

mov dx,offset hello_message

mov ah,09h

int 21h

mov ax,4C00h

int 21h

main endp

end main

# Line-by-Line Explanation

## Line 1

org 100h

Explanation

ORG stands for Origin.

It tells the assembler that the program will start from memory location 100H.

All COM programs in EMU8086 start from address 100H.

## Line 2

.data

Explanation

This is the Data Section.

All variables, messages and strings are declared here.

## Line 3

hello_message db 'Hello',' World',0dh,0ah,'$'

### Explanation

A variable named hello_message is created.

DB means Define Byte.

The string stored is:

Hello World

### Meaning of each part

Hello

Stores the word Hello.

World

Stores the word World.

0DH

Carriage Return

Moves the cursor to the beginning of the next line.

0AH

Line Feed

Moves the cursor down one line.

Together

0DH 0AH

create a new line.

$

Marks the end of the string.

DOS Function 09H keeps printing characters until it finds $.

Without $, random garbage characters may appear.

## Line 4

.code

### Explanation

This is the Code Section.

All executable instructions are written here.

## Line 5

main proc

### Explanation

Starts the main procedure.

Program execution begins from here.

## Line 6

mov ax,@data

### Explanation

Loads the address of the Data Segment into AX.

The CPU now knows where the variables are located.

## Line 7

mov ds,ax

### Explanation

Copies the address stored in AX into the DS register.

DS now points to the Data Segment.

Without this instruction, variables cannot be accessed correctly.

## Line 8

mov dx,offset hello_message

### Explanation

OFFSET returns the starting memory address of hello_message.

That address is stored in DX.

Function 09H uses DX to know where the string starts.

## Line 9

mov ah,09h

### Explanation

AH stores the DOS function number.

Function 09H means:

Display a complete string on the screen.

## Line 10

int 21h

### Explanation

Calls DOS Interrupt 21H.

DOS reads the function number from AH.

Since AH = 09H,

DOS prints the string whose address is stored in DX.

## Line 11

mov ax,4C00h

### Explanation

Loads Function 4CH into AH.

Function 4CH terminates the program.

## Line 12

int 21h

### Explanation

Returns control back to DOS.

The program finishes execution.

## Line 13

main endp

### Explanation

Marks the end of the Main Procedure.

## Line 14

end main

### Explanation

Tells the assembler that the source program ends here.

# Program Flow

Program Starts

↓

Load Data Segment

↓

Load Address of String

↓

AH = 09H

↓

INT 21H

↓

Hello World is displayed

↓

AH = 4CH

↓

INT 21H

↓

Program Ends

# Output

Hello World

# Important Points

• Function 09H displays strings.

• String must end with $.

• OFFSET gives the address of a variable.

• DX stores the address of the string.

• DS must point to the Data Segment.

• Every program should end with

mov ax,4C00h

int 21h

# Viva Questions

Q1. Why do we use ORG 100H?

Answer:

Because COM programs begin execution from memory location 100H.

Q2. Why is '$' placed at the end of the string?

Answer:

Because Function 09H prints characters until it finds the $ symbol.

Q3. Which register stores the address of the string?

Answer:

DX Register.

Q4. Which interrupt displays the string?

Answer:

INT 21H with AH = 09H.

# Exercise 2.1 : Output of Activity 2.1

## Output

Hello World

### Explanation

The program prints the string stored in hello_message.

Because 0DH and 0AH are included, the cursor moves to the next line after printing.

# Exercise 2.2 : Print Name and Date of Birth

## Objective

To learn how to display more than one string by calling Function 09H multiple times.

## Program

org 100h

.data

n db 'Muhammad Mehrab Ali',0dh,0ah,'$'

date_of_birth db '20 January,2006',0dh,0ah,'$'

.code

main proc

mov ax,@data

mov ds,ax

mov dx,offset n

mov ah,09h

int 21h

mov dx,offset date_of_birth

mov ah,09h

int 21h

mov ax,4C00h

int 21h

main endp

end main

# Line-by-Line Explanation

The first four instructions are exactly the same as the Hello World program.

The only difference is that two strings are declared.

n

stores your name.

date_of_birth

stores your date of birth.

The first

mov dx,offset n

points DX to your name.

INT 21H prints the name.

Next,

mov dx,offset date_of_birth

changes DX so that it points to the second string.

Another

int 21h

prints your date of birth.

This shows that Function 09H can print multiple strings by changing the address stored in DX.

# Output

Muhammad Mehrab Ali

20 January, 2006

# Important Points

• Multiple strings can be declared inside the .DATA section.

• Function 09H can be called as many times as needed.

• Only the address in DX needs to be changed before each call.

# Exercise 2.3 – Program 1 (Hello World)

## Program

org 100h

.data

msg db "Hello, World",0dh,0ah,'$'

.code

mov ax,@data

mov ds,ax

mov dx,offset msg

mov ah,09h

int 21h

mov ax,4C00h

int 21h

### Explanation

This program works exactly like Activity 2.1.

The only difference is that the variable name is msg instead of hello_message.

## Output

Hello, World

# Exercise 2.3 – Program 2 (Addition)

## Program

org 100h

mov ax,2

mov bx,2

add ax,bx

mov dl,al

add dl,30h

mov ah,02h

int 21h

mov ax,4C00h

int 21h

# Line-by-Line Explanation

mov ax,2

Stores 2 in AX.

mov bx,2

Stores 2 in BX.

add ax,bx

Adds AX and BX.

Result:

AX = 4

mov dl,al

Copies the lower 8 bits of AX into DL.

add dl,30h

Converts the number into its ASCII code.

Without this instruction, the number would not display correctly.

mov ah,02h

Function 02H displays one character.

int 21h

Displays the character stored in DL.

## Output

4

# Exercise 2.3 – Program 3 (Subtraction)

## Program

org 100h

mov ax,19

sub ax,10

mov dl,al

add dl,30h

mov ah,02h

int 21h

mov ax,4C00h

int 21h

# Line-by-Line Explanation

mov ax,19

Stores 19 in AX.

sub ax,10

Subtracts 10.

Result:

AX = 9

mov dl,al

Copies the result into DL.

add dl,30h

Converts the numeric value into its ASCII equivalent.

mov ah,02h

Selects DOS Function 02H.

int 21h

Displays the digit.

## Output

9

# Lab 2 Summary

After completing Lab 2, you should know:

- Basic structure of an Assembly program.

- How to declare variables using DB.

- How to print strings using AH = 09H.

- How to display one character using AH = 02H.

- How to use OFFSET.

- How to initialize the Data Segment using MOV AX,@DATA and MOV DS,AX.

- How to perform simple addition and subtraction.

- Why ADD DL,30H is required before displaying numeric values.

- How to terminate a program using MOV AX,4C00H and INT 21H.

# Lab 3

# Activity 3.1 – Addition of Binary Digits

## Objective

The purpose of this program is to learn how to add two binary numbers using the ADD instruction and understand how binary values are stored inside CPU registers.

# Program

org 100h

.code

main proc

mov al,01001011b

mov bl,00000011b

add al,bl

mov dl,al

mov ah,02h

int 21h

mov ax,4C00h

int 21h

main endp

end main

# Line-by-Line Explanation

## Line 1

org 100h

### Explanation

The program starts at memory location 100H.

All COM programs begin from this address.

## Line 2

.code

### Explanation

This is the code section where executable instructions are written.

## Line 3

main proc

### Explanation

Marks the beginning of the main procedure.

Program execution starts here.

## Line 4

mov al,01001011b

### Explanation

Loads the binary number

01001011

into register AL.

The letter b at the end means the number is written in Binary.

Binary

01001011

is equal to

75 (Decimal)

## Line 5

mov bl,00000011b

### Explanation

Loads another binary number into BL.

00000011

equals

3 (Decimal)

## Line 6

add al,bl

### Explanation

Adds BL to AL.

Calculation

75 + 3 = 78

After execution

AL = 78

## Line 7

mov dl,al

### Explanation

Copies the result from AL into DL because Function 02H displays the character stored in DL.

## Line 8

mov ah,02h

### Explanation

Selects DOS Function 02H.

Function 02H displays one character.

## Line 9

int 21h

### Explanation

Calls DOS Interrupt 21H.

The character stored in DL is displayed.

Note: Since the result is a binary value (78), it represents the ASCII character N. Therefore, this program displays N, not the number 78.

If you want to display the decimal digit correctly, you must first convert the number into ASCII (for single-digit values, by adding 30H).

## Line 10

mov ax,4C00h

### Explanation

Loads DOS Function 4CH.

This function terminates the program.

## Line 11

int 21h

### Explanation

Returns control to DOS and ends the program.

# Program Flow

Program Starts

↓

Load Binary Number into AL

↓

Load Binary Number into BL

↓

ADD AL,BL

↓

Copy Result to DL

↓

AH = 02H

↓

INT 21H

↓

Display Character

↓

Program Ends

# Output

N

(ASCII character for decimal value 78.)

# Important Points

• Binary numbers end with B.

• ADD stores the result in the destination register.

• Function 02H displays one character only.

• DL must contain the character to display.

• Binary values are stored inside registers exactly like decimal values.

# Activity 3.2 – Loop Program

## Objective

The purpose of this program is to understand the LOOP instruction and how the CPU repeatedly executes the same block of code.

# Program

.code

main proc

mov cl,12

mov bl,65

AGN:

inc bl

mov dl,bl

mov ah,02h

int 21h

loop AGN

mov ax,4C00h

int 21h

main endp

end main

# Line-by-Line Explanation

## Line 1

mov cl,12

### Explanation

Stores 12 inside CL.

CL acts as the loop counter.

The loop will execute 12 times.

## Line 2

mov bl,65

### Explanation

Stores decimal value 65 inside BL.

ASCII code

65 = A

## Line 3

AGN:

### Explanation

This is a Label.

Whenever the LOOP instruction executes, the program jumps back to this label.

## Line 4

inc bl

### Explanation

Increases BL by 1.

First iteration

65 → 66

ASCII

66 = B

Next iteration

67 = C

68 = D

69 = E

and so on.

## Line 5

mov dl,bl

### Explanation

Copies BL into DL.

Function 02H always displays the character stored in DL.

## Line 6

mov ah,02h

### Explanation

Selects DOS Function 02H.

## Line 7

int 21h

### Explanation

Displays the character stored in DL.

## Line 8

loop AGN

### Explanation

Automatically decreases the loop counter by 1.

If the counter is not zero, execution jumps back to the label AGN.

When the counter becomes zero, the loop stops.

## Line 9

mov ax,4C00h

Ends the program.

## Line 10

int 21h

Returns control to DOS.

# Program Flow

Start

↓

CL = 12

BL = 65

↓

Increase BL

↓

Display Character

↓

Decrease Counter

↓

Counter = 0 ?

↓          ↓

No          Yes

↓            ↓

Repeat     End

# Output

BCDEFGHIJKLM

# Important Points

• LOOP automatically decreases the counter.

• LOOP uses the CX register (or CL in this program).

• INC increases a value by one.

• Labels tell the CPU where to jump.

# Viva Questions

Q1. Which register controls the loop?

Answer:

CX (or CL)

Q2. Which instruction increases a value by one?

Answer:

INC

Q3. What is a Label?

Answer:

A label is the name of a location in the program to which execution can jump.

# Activity 3.3 – Display Character 'a'

## Objective

To learn how to display a single character on the screen using INT 21H Function 02H.

# Program

org 100h

.data

.code

main proc

mov dl,'a'

mov ah,02h

int 21h

mov ax,4C00h

int 21h

main endp

end main

# Line-by-Line Explanation

## Line 1

mov dl,'a'

### Explanation

Stores the character

a

inside register DL.

## Line 2

mov ah,02h

### Explanation

Selects DOS Function 02H, which displays one character.

## Line 3

int 21h

### Explanation

Calls DOS.

DOS prints the character stored in DL.

## Line 4

mov ax,4C00h

Loads Function 4CH to terminate the program.

## Line 5

int 21h

Ends the program.

# Program Flow

Program Starts

↓

Store 'a' in DL

↓

AH = 02H

↓

INT 21H

↓

Display 'a'

↓

End Program

# Output

a

# Important Points

• Function 02H displays only one character.

• The character must be stored in DL.

• If you want to display a string, use AH = 09H, not 02H.

# Input Character Example

## Objective

The objective of this program is to learn how to take a single character as input from the keyboard and understand how the entered character is stored inside the CPU registers. Complete Code

org 100h

mov ah,01h

int 21h

mov dl,0Dh

mov ah,02h

int 21h

mov dl,0Ah

mov ah,02h

int 21h

# Program Working

- The program waits for the user to press a key.

- The character entered by the user is automatically stored in the AL register.

- The program moves the cursor to the next line using Carriage Return (0DH) and Line Feed (0AH).

# Line-by-Line Explanation

### Line 1

org 100h

Explanation

Tells the assembler that the program starts from memory location 100H.

### Line 2

mov ah,01h

Explanation

Loads Function 01H into register AH.

Function 01H is used to take one character input from the keyboard.

### Line 3

int 21h

Explanation

Calls DOS Interrupt 21H.

The program waits until the user presses any key.

The entered character is automatically stored in the AL register.

For example:

If the user enters

A

then

AL = 'A'

If the user enters

7

then

AL = '7'

### Line 4

mov dl,0Dh

Explanation

Loads ASCII code 0DH into register DL.

0DH is called Carriage Return (CR).

It moves the cursor to the beginning of the current line.

### Line 5

mov ah,02h

Explanation

Selects DOS Function 02H, which is used to display one character.

### Line 6

int 21h

Explanation

Displays the Carriage Return character.

The cursor moves to the beginning of the line.

### Line 7

mov dl,0Ah

Explanation

Loads ASCII code 0AH into DL.

0AH is called Line Feed (LF).

It moves the cursor down by one line.

### Line 8

mov ah,02h

Explanation

Again selects Function 02H.

### Line 9

int 21h

Explanation

Displays the Line Feed character.

Now the cursor moves to a completely new line.

# Program Flow

Program Starts

↓

AH = 01H

↓

INT 21H

↓

User Enters a Character

↓

Character Stored in AL

↓

Display Carriage Return (0DH)

↓

Display Line Feed (0AH)

↓

Cursor Moves to New Line

↓

Program Ends

# Output

Example

Input:

M

Output:

(Cursor moves to the next line)

This program does not display the entered character again. It only accepts the input and moves the cursor to the next line.

# Important Points

- Function 01H is used for keyboard input.

- The entered character is automatically stored in AL.

- Function 02H displays one character.

- 0DH means Carriage Return.

- 0AH means Line Feed.

- Both 0DH and 0AH are used together to move the cursor to a new line.

Activity 3.4 – Input and Display Character

## Objective

The objective of this program is to take one character from the user and display the same character back on the screen.

This activity teaches how to:

- Display messages.

- Take keyboard input.

- Store input in a register.

- Display the stored character.

# Complete Code

org 100h

.data

inMsg db "Enter a character ",'$'

newline db 0dh,0ah,'$'

outMsg db "User entered character ",'$'

.code

main proc

mov ax,@data

mov ds,ax

mov dx,offset inMsg

mov ah,09h

int 21h

mov ah,01h

int 21h

mov bl,al

mov dx,offset newline

mov ah,09h

int 21h

mov dx,offset outMsg

mov ah,09h

int 21h

mov dl,bl

mov ah,02h

int 21h

mov ax,4C00h

int 21h

main endp

end main

# Program Working

- Displays the message "Enter a character".

- Waits for the user to press a key.

- Stores the entered character in AL.

- Copies the character into BL for safe storage.

- Moves to the next line.

- Displays the message "User entered character".

- Displays the character entered by the user.

- Ends the program.

# Line-by-Line Explanation

### Data Section

inMsg db "Enter a character ",'$'

Stores the first message.

newline db 0dh,0ah,'$'

Stores a new line.

outMsg db "User entered character ",'$'

Stores the second message.

### Initialize Data Segment

mov ax,@data

mov ds,ax

These instructions initialize the Data Segment so that variables and strings can be accessed correctly.

### Display First Message

mov dx,offset inMsg

mov ah,09h

int 21h

Displays

Enter a character

### Take Input

mov ah,01h

int 21h

Waits for the user to press a key.

The entered character is stored in AL.

### Save the Character

mov bl,al

Copies the character into BL.

This is done because later instructions may change AL.

### Move to Next Line

mov dx,offset newline

mov ah,09h

int 21h

Prints a new line using 0DH and 0AH.

### Display Second Message

mov dx,offset outMsg

mov ah,09h

int 21h

Displays

User entered character

### Display the Character

mov dl,bl

mov ah,02h

int 21h

Copies the stored character into DL.

Function 02H displays the character.

### End Program

mov ax,4C00h

int 21h

Ends the program and returns control to DOS.

# Program Flow

Program Starts

↓

Initialize Data Segment

↓

Display "Enter a character"

↓

Take Input from Keyboard

↓

Store Character in BL

↓

Print New Line

↓

Display "User entered character"

↓

Display Stored Character

↓

End Program

# Example Output

Enter a character: M

User entered character: M

Task 3.1 – Convert Lowercase Letter to Uppercase

## Objective

The objective of this program is to input a lowercase letter from the user, convert it into an uppercase letter using its ASCII value, and display the uppercase letter on the screen.

This program also helps us understand:

- ASCII values

- Character conversion

- Keyboard input

- Character output

# Complete Code

org 100h

.data

msg1 db 'Enter a lowercase letter:','$'

msg2 db 0Dh,0Ah,'Uppercase letter:','$'

.code

main proc

mov ax,@data

mov ds,ax

mov dx,offset msg1

mov ah,09h

int 21h

mov ah,01h

int 21h

sub al,32

mov bl,al

mov dx,offset msg2

mov ah,09h

int 21h

mov dl,bl

mov ah,02h

int 21h

mov ax,4C00h

int 21h

main endp

end main

# Program Working

- Displays the message "Enter a lowercase letter:"

- Takes one character from the keyboard.

- Stores the character in AL.

- Subtracts 32 from its ASCII value.

- The lowercase letter becomes uppercase.

- Displays the message "Uppercase letter:"

- Displays the converted uppercase character.

- Ends the program.

# Line-by-Line Explanation

### Line 1

org 100h

Explanation

Tells the assembler that the program starts at memory location 100H.

### Data Section

.data

Stores all variables and messages.

msg1 db 'Enter a lowercase letter:','$'

Stores the first message.

msg2 db 0Dh,0Ah,'Uppercase letter:','$'

Stores a new line followed by the second message.

### Code Section

.code

Contains executable instructions.

### Start Main Procedure

main proc

Marks the beginning of the program.

### Initialize Data Segment

mov ax,@data

mov ds,ax

Initializes the Data Segment so variables can be accessed.

### Display First Message

mov dx,offset msg1

mov ah,09h

int 21h

Displays:

Enter a lowercase letter:

### Input Character

mov ah,01h

int 21h

Waits for the user to press a key.

The entered character is stored in AL.

Example

If the user enters

a

then

AL = 'a'

### Convert Lowercase to Uppercase

sub al,32

Subtracts 32 from the ASCII value.

ASCII Table

a = 97

A = 65

97 - 32 = 65

Therefore

a → A

b → B

c → C

### Save the Character

mov bl,al

Copies the uppercase letter into BL.

### Display Second Message

mov dx,offset msg2

mov ah,09h

int 21h

Displays

Uppercase letter:

### Display the Character

mov dl,bl

mov ah,02h

int 21h

Displays the uppercase letter stored in BL.

### End Program

mov ax,4C00h

int 21h

Ends the program.

# Program Flow

Program Starts

↓

Initialize Data Segment

↓

Display "Enter a lowercase letter"

↓

Take Input

↓

Subtract 32 from ASCII Value

↓

Store Result in BL

↓

Display "Uppercase letter"

↓

Display Uppercase Character

↓

Program Ends

# Example Output

Enter a lowercase letter: d

Uppercase letter: D

# Task 3.2 – Display a Character 10 Times

## Objective

The objective of this program is to input one character from the user and display it 10 times, each on a new line, using the LOOP instruction.

# Complete Code

org 100h

.data

msg db 'Enter a character: $'

.code

main proc

mov ax,@data

mov ds,ax

mov dx,offset msg

mov ah,09h

int 21h

mov ah,01h

int 21h

mov bl,al

mov cx,10

repeat:

mov dl,0Dh

mov ah,02h

int 21h

mov dl,0Ah

mov ah,02h

int 21h

mov dl,bl

mov ah,02h

int 21h

loop repeat

mov ax,4C00h

int 21h

main endp

end main

# Program Working

- Displays "Enter a character:"

- Takes one character as input.

- Stores the character in BL.

- Sets CX = 10.

- Prints a new line.

- Displays the character.

- LOOP repeats until CX becomes zero.

- Ends the program.

# Line-by-Line Explanation

### Display Message

mov dx,offset msg

mov ah,09h

int 21h

Displays:

Enter a character:

### Input Character

mov ah,01h

int 21h

Reads one character.

Character is stored in AL.

### Save Character

mov bl,al

Stores the character in BL for repeated use.

### Loop Counter

mov cx,10

Sets the loop counter to 10.

The loop will execute ten times.

### Label

repeat:

Marks the beginning of the loop.

### New Line

mov dl,0Dh

mov ah,02h

int 21h

mov dl,0Ah

mov ah,02h

int 21h

Moves the cursor to the next line.

### Display Character

mov dl,bl

mov ah,02h

int 21h

Displays the stored character.

### Repeat Loop

loop repeat

Decreases CX by 1.

If CX ≠ 0, execution jumps back to repeat.

When CX = 0, the loop stops.

### End Program

mov ax,4C00h

int 21h

Terminates the program.

# Program Flow

Program Starts

↓

Display Message

↓

Take Input

↓

Store Character in BL

↓

CX = 10

↓

Print New Line

↓

Display Character

↓

Decrease CX

↓

CX = 0 ?

↓        ↓

No        Yes

↓          ↓

Repeat     End

# Example Output

If the user enters

A

Output

A

A

A

A

A

A

A

A

A

A

# Task 3.3 – Display Uppercase Letters (A–Z) Using LOOP

## Objective

The objective of this program is to display all uppercase English letters from A to Z, with each letter printed on a new line using the LOOP instruction.

This program helps us understand:

- ASCII characters

- LOOP instruction

- INC instruction

- Character display using INT 21H

- New line using 0DH and 0AH

# Complete Code

org 100h

.data

.code

main proc

mov cx,26

mov bl,'A'

print:

mov dl,bl

mov ah,02h

int 21h

mov dl,0Dh

mov ah,02h

int 21h

mov dl,0Ah

mov ah,02h

int 21h

inc bl

loop print

mov ax,4C00h

int 21h

main endp

end main

# Program Working

- The loop counter is initialized to 26.

- BL is loaded with the ASCII value of 'A'.

- The character stored in BL is displayed.

- A new line is printed.

- BL is increased by one using INC.

- LOOP decreases CX by one.

- The loop continues until all 26 letters have been displayed.

- The program terminates.

# Line-by-Line Explanation

### Line 1

org 100h

Explanation

Tells the assembler that the program starts from memory address 100H.

### Line 2

.data

Explanation

This is the Data Section.

No variables are declared in this program.

### Line 3

.code

Explanation

This section contains executable instructions.

### Line 4

main proc

Explanation

Marks the beginning of the main procedure.

### Line 5

mov cx,26

Explanation

Stores 26 in register CX.

Since there are 26 uppercase English letters, the loop will execute 26 times.

### Line 6

mov bl,'A'

Explanation

Stores the ASCII value of uppercase A in BL.

ASCII value

A = 65

### Line 7

print:

Explanation

This is a label.

The LOOP instruction returns to this label after each iteration.

### Line 8

mov dl,bl

Explanation

Copies the current letter from BL into DL.

DOS Function 02H always displays the character stored in DL.

### Line 9

mov ah,02h

Explanation

Selects DOS Function 02H.

Function 02H displays one character.

### Line 10

int 21h

Explanation

Displays the character stored in DL.

### Line 11

mov dl,0Dh

Explanation

Loads Carriage Return into DL.

Moves the cursor to the beginning of the next line.

### Line 12

mov ah,02h

Explanation

Selects Function 02H again.

### Line 13

int 21h

Explanation

Displays Carriage Return.

### Line 14

mov dl,0Ah

Explanation

Loads Line Feed into DL.

Moves the cursor one line downward.

### Line 15

mov ah,02h

Explanation

Again selects Function 02H.

### Line 16

int 21h

Explanation

Displays Line Feed.

Now the cursor moves to the next line.

### Line 17

inc bl

Explanation

Increases BL by 1.

Example:

A → B

B → C

C → D

This continues until Z.

### Line 18

loop print

Explanation

The LOOP instruction:

- Decreases CX by 1.

- Checks whether CX is zero.

- If CX is not zero, execution jumps back to the label print.

### Line 19

mov ax,4C00h

Explanation

Loads DOS Function 4CH into AX.

This function terminates the program.

### Line 20

int 21h

Explanation

Returns control to DOS.

### Line 21

main endp

Marks the end of the Main Procedure.

### Line 22

end main

Marks the end of the Assembly program.

# Program Flow

Program Starts

↓

CX = 26

↓

BL = 'A'

↓

Display Letter

↓

Print New Line

↓

INC BL

↓

LOOP

↓

CX = 0 ?

↓         ↓

No         Yes

↓           ↓

Repeat      End Program

# Output

A

B

C

D

E

F

G

H

I

J

K

L

M

N

O

P

Q

R

S

T

U

V

W

X

Y

Z

# Important Points

- LOOP automatically decreases CX.

- CX controls the number of repetitions.

- INC increases a register by one.

- Function 02H displays one character.

- BL stores the current letter.

- DL stores the character before displaying it.

- ASCII letters are stored in sequence, so increasing BL prints the next letter.

## Important Registers

- AX → Accumulator Register

- BX → Base Register

- CX → Counter Register

- DX → Data Register

## DOS Interrupt Functions

| Function | Purpose |
| --- | --- |
| AH = 01H | Input one character |
| AH = 02H | Display one character |
| AH = 09H | Display a string |
| AH = 4CH | Exit program |

## Common Instructions

- MOV → Copy data

- ADD → Addition

- SUB → Subtraction

- INC → Increase by 1

- DEC → Decrease by 1

- LOOP → Repeat instructions

- INT → Call DOS interrupt

## ASCII Values

| Character | ASCII |
| --- | --- |
| 0 | 48 |
| A | 65 |
| Z | 90 |
| a | 97 |
| z | 122 |

## Remember

- Use AH = 09H for strings.

- Use AH = 02H for a single character.

- Use AH = 01H for keyboard input.

- Always initialize the Data Segment using:

mov ax,@data

mov ds,ax

- End every program using:

mov ax,4C00h

int 21h

# Revision Notes (Lab 1 & Lab 2)

### Q1. What is Assembly Language?

Answer:
Assembly Language is a low-level programming language that uses mnemonic instructions (such as MOV, ADD, SUB) to communicate directly with the microprocessor.

### Q2. Why is Assembly Language faster than High-Level Languages?

Answer:
Because Assembly Language communicates directly with the CPU and requires very little translation.

### Q3. What is Machine Language?

Answer:
Machine Language is the language understood directly by the processor. It consists of only binary digits (0 and 1).

### Q4. What is a Microprocessor?

Answer:
A Microprocessor is the brain of the computer that performs arithmetic, logical, and control operations.

### Q5. What is the Intel 8086?

Answer:
The Intel 8086 is a 16-bit microprocessor introduced by Intel. It is widely used for learning Assembly Language programming.

### Q6. What is EMU8086?

Answer:
EMU8086 is an emulator used to write, assemble, debug, and execute Assembly Language programs for the Intel 8086 processor.

### Q7. What is DOS?

Answer:
DOS (Disk Operating System) is an operating system that provides services for running programs using interrupts such as INT 21H.

## Program Structure

### Q8. What is ORG 100H?

Answer:
ORG stands for Origin. It tells the assembler that the program starts from memory address 100H.

### Q9. Why do COM programs start at 100H?

Answer:
Because the first 256 bytes are reserved for the Program Segment Prefix (PSP). Therefore, COM programs begin execution from address 100H.

### Q10. What is the .DATA section?

Answer:
The .DATA section is used to declare variables, constants, and strings.

### Q11. What is the .CODE section?

Answer:
The .CODE section contains executable instructions.

### Q12. What is MAIN PROC?

Answer:
MAIN PROC marks the beginning of the main procedure.

### Q13. What is MAIN ENDP?

Answer:
MAIN ENDP marks the end of the procedure.

### Q14. What is END MAIN?

Answer:
It tells the assembler where the program ends.

## Registers

### Q15. What is a Register?

Answer:
Registers are small, high-speed memory locations inside the CPU used for temporary data storage.

### Q16. What is AX?

Answer:
AX is the Accumulator Register used for arithmetic, logical, multiplication, division, and data movement operations.

### Q17. What is AH?

Answer:
AH is the higher 8 bits of the AX register.

### Q18. What is AL?

Answer:
AL is the lower 8 bits of the AX register.

### Q19. What is BX?

Answer:
BX is the Base Register used for storing data and memory addresses.

### Q20. What is CX?

Answer:
CX is the Counter Register mainly used with the LOOP instruction.

### Q21. What is DX?

Answer:
DX is the Data Register mainly used for input/output operations.

## Instructions

### Q22. What is MOV?

Answer:
MOV copies data from the source to the destination.

### Q23. What is ADD?

Answer:
ADD performs addition.

### Q24. What is SUB?

Answer:
SUB performs subtraction.

### Q25. What is INC?

Answer:
INC increases a value by one.

### Q26. What is DEC?

Answer:
DEC decreases a value by one.

### Q27. What is LOOP?

Answer:
LOOP decreases CX by one and repeats execution until CX becomes zero.

## Data Directives

### Q28. What does DB mean?

Answer:
DB stands for Define Byte.

It is used to declare variables and strings.

Example:

msg db 'Hello','$'

### Q29. What is a String?

Answer:
A string is a collection of characters.

Example:

Hello World

### Q30. Why is '$' used at the end of a string?

Answer:
Because DOS Function 09H prints characters until it finds the '$' symbol.

### Q31. What is OFFSET?

Answer:
OFFSET returns the starting memory address of a variable or string.

### Q32. What is @DATA?

Answer:
@DATA represents the address of the Data Segment.

### Q33. Why do we write

mov ax,@data

mov ds,ax

Answer:
To initialize the Data Segment Register (DS) so that variables in the .DATA section can be accessed.

## Interrupts

### Q34. What is an Interrupt?

Answer:
An interrupt is a signal that requests the CPU to perform a specific service.

### Q35. What is INT 21H?

Answer:
INT 21H is a DOS interrupt that provides various input, output, and system functions.

### Q36. What does AH = 01H do?

Answer:
Inputs one character from the keyboard.

### Q37. What does AH = 02H do?

Answer:
Displays one character stored in the DL register.

### Q38. What does AH = 09H do?

Answer:
Displays a string ending with '$'.

### Q39. What does AH = 4CH do?

Answer:
Terminates the program and returns control to DOS.

## ASCII

### Q40. What is ASCII?

Answer:
ASCII (American Standard Code for Information Interchange) assigns a unique numeric value to every character.

Examples:

- A = 65

- Z = 90

- a = 97

- z = 122

- 0 = 48

- 9 = 57

### Q41. Why do we subtract 32?

Answer:
To convert a lowercase letter into an uppercase letter.

### Q42. Why do we add 30H?

Answer:
To convert a single-digit numeric value into its ASCII character before displaying it.

# Common Programming Mistakes

Students commonly make these mistakes while writing Assembly Language programs:

### 1. Forgetting ORG 100H

Without ORG 100H, COM programs may not execute correctly.

### 2. Forgetting .DATA or .CODE

Always separate variables and executable instructions into their proper sections.

### 3. Forgetting to Initialize the Data Segment

Always write:

mov ax,@data

mov ds,ax

before accessing variables.

### 4. Forgetting '$' at the End of a String

Without '$', Function 09H may print unwanted or garbage characters.

### 5. Using Function 02H to Display a String

AH = 02H displays only one character.

Use AH = 09H to display strings.

### 6. Forgetting OFFSET

Always use OFFSET before the name of a string when using Function 09H.

Example:

mov dx,offset msg

### 7. Displaying Numbers Without ASCII Conversion

A number should be converted into ASCII before displaying.

Example:

add dl,30h

### 8. Forgetting INT 21H

The selected DOS function will not execute until INT 21H is called.

### 9. Forgetting to End the Program

Always terminate programs with:

mov ax,4C00h

int 21h

### 10. Incorrect LOOP Counter

Always initialize CX before using LOOP.

Example:

mov cx,10

### 11. Forgetting to Save User Input

Store the entered character in another register (such as BL) if it will be used later.

### 12. Mixing Character and String Functions

- AH = 02H → Single Character

- AH = 09H → Complete String

# 3. Quick Revision Sheet

## Basic Program Structure

org 100h

.data

.code

main proc

mov ax,@data

mov ds,ax

; Program Instructions

mov ax,4C00h

int 21h

main endp

end main

## General Purpose Registers

| Register | Purpose |
| --- | --- |
| AX | Accumulator Register |
| BX | Base Register |
| CX | Counter Register |
| DX | Data Register |

## Important DOS Functions (INT 21H)

| AH Value | Function |
| --- | --- |
| 01H | Input one character |
| 02H | Display one character |
| 09H | Display a string |
| 4CH | Exit the program |

## Frequently Used Instructions

| Instruction | Purpose |
| --- | --- |
| MOV | Copy data |
| ADD | Addition |
| SUB | Subtraction |
| INC | Increase by 1 |
| DEC | Decrease by 1 |
| LOOP | Repeat instructions |
| INT | Call DOS interrupt |
| DB | Define Byte |
| OFFSET | Get memory address |

## ASCII Values to Remember

| Character | ASCII |
| --- | --- |
| 0 | 48 |
| 9 | 57 |
| A | 65 |
| Z | 90 |
| a | 97 |
| z | 122 |

## New Line Characters

| Code | Meaning |
| --- | --- |
| 0DH | Carriage Return (CR) |
| 0AH | Line Feed (LF) |

Use 0DH and 0AH together to move the cursor to the beginning of the next line.

## Important Rules

- Every COM program begins with ORG 100H.

- Initialize the Data Segment before using variables.

- Every string displayed with Function 09H must end with '$'.

- Use OFFSET to obtain the address of a string.

- Store characters in DL before displaying them.

- Store keyboard input in AL.

- Initialize CX before using the LOOP instruction.

- End every Assembly program with:

mov ax,4C00h

int 21h

## Why do we use Two's Complement?

Computers cannot directly understand negative numbers. They only store 0s and 1s.

So instead of storing -5, the computer stores the two's complement of 5.

This allows the computer to use the same addition circuit for both addition and subtraction.

Instead of building:

- One circuit for addition

- Another circuit for subtraction

The CPU only needs one adder.

That's why we write:

A−B  = A+(−B)

Here (-B) means the two's complement of B.

# How to find Two's Complement

Suppose

B = 00001111

### Step 1: Find 1's complement (invert every bit)

00001111

↓↓↓↓↓↓↓↓

11110000

### Step 2: Add 1

11110000

+00000001

----------

11110001

So

-00001111 = 11110001

# Now your example

You gave

A = 00001111

B = 11110000

First we must know what B actually represents.

### Binary 11110000

Since the first bit is 1, this is already a negative number (in two's complement).

Let's find its decimal value.

Invert bits

11110000

↓↓↓↓↓↓↓↓

00001111

Add 1

00001111

+00000001

----------

00010000

00010000 = 16

Therefore

11110000 = -16

So your numbers are

A = 15

B = -16

# What is A − B ?

Using the formula

A − B = A + (-B)

We have

15 − (-16)

which becomes

15 + 16

=31

Binary:

00001111

+00010000

----------

00011111

31 decimal

# What if B were +16?

Suppose

A = 00011111   (31)

B = 00010000   (16)

We want

31 −16

Instead of subtracting, we calculate

31 + (-16)

Find two's complement of B.

00010000

Invert

11101111

Add 1

11110000

Now add

00011111

11110000

---------

1 00001111

Ignore the carry

00001111

which is

15

Correct!

# Important Rule

### To perform subtraction

A − B

Do this:

- Find two's complement of B.

- Add it to A.

- Ignore the carry (if there is one).

## Your specific values

A = 00001111 = 15

B = 11110000 = -16

So

A − B

becomes

15 − (-16)

which is

15 + 16

=31

Binary result:

00011111

LAB 4:

ACTIVITY 4.3: ADDITION OF TWO SINGLE DIGITS NUMBERS?

Answer:

org 100h

.data

num1 db 0

num2 db 0

ans db 0

msg1 db 'Enter no 1:','$'

msg2 db 0dh,0ah,'Enter no 2:','$'

msg3 db 0dh,0ah,'The answer is:','$'

.code

main proc

mov ax,@data

mov ds,ax

mov dx,offset msg1

mov ah,9h

int 21h

mov ah,1

int 21h

sub al,30h

mov num1,al

mov bl,num1

mov ax,@data

mov ds,ax

mov dx,offset msg2

mov ah,9h

int 21h

mov ah,1

int 21h

sub al,30h

mov num2,al

add bl,num2

add bl,30h

mov ans,bl

mov ax,@data

mov ds,ax

mov ax,offset msg3

mov ah,9h

int 21h

mov dl,ans

mov ah,2h

int 21h

mov ax,4c00h

int 21h

main endp

end main

ACTIVITY 4.4: ADDITION OF TWO MULTI-DIGITS NUMBERS?

Answer:

include emu8086.inc

.data

num1 dw 0

num2 dw 0

.code

main proc

call scan_num

mov num1, cx

call scan_num

mov num2, cx

mov ax, num1

add ax, num2

call print_num

ret

main endp

DEFINE_PRINT_NUM

DEFINE_PRINT_NUM_UNS

DEFINE_SCAN_NUM

END main

Exercise 4.1: Modify above program to format the output.

Enter first number:

Enter second number:

The sum of two number is :

Solution:

include 'emu8086.inc'

org 100h

.data

num1 db 'Enter first number: ','$'

num2 db 0dh,0ah,'Enter second number: ','$'

num3 db 0dh,0ah,'The answer is: ','$'

ans db 0

.code

main proc

gotoxy 5,3

mov ax,@data

mov ds,ax

mov dx,offset num1

mov ah,9h

int 21h

mov ah,1

int 21h

sub al,30h

mov num1,al

mov bl,num1

mov ax,@data

mov ds,ax

mov dx,offset num2

mov ah,9h

int 21h

mov ah,1h

int 21h

sub al,30h

mov num2,al

add bl,num2

add bl,30h

mov ans,bl

mov ax,@data

mov ds,ax

mov dx,offset num3

mov ah,9

int 21h

mov dl,ans

mov ah,2h

int 21h

mov ax,4c00h

int 21h

main endp

end main

EXERCISE 4.2

Write a program that reads five unsigned integers from user and display sum of those five integers on the screen.

Solution

include 'emu8086.inc'

org 100h

.data

ans dw 0

num1 db 'Enter five numbers: ',0dh,0ah,'$'

num2 db 'The sum is: ','$'

.code

main proc

mov cx,5

sum:

push cx

mov ax,@data

mov ds,ax

mov dx,offset num1

mov ah,9h

int 21h

call scan_num

add ans,cx

pop cx

loop sum

printn ' '

mov ax,@data

mov ds,ax

mov dx,offset num2

mov ah,9h

int 21h

mov ax,ans

call print_num

main endp

define_scan_num

define_print_num

define_print_num_uns

end main

EXERCISE 4.3

Write a program using instruction shown in table and fill in the table with the offsets of the instructions in the code segment.

Solution:

| Instruction | Segment and Offset |
| --- | --- |
| mov ax, number | 0700, 0106 |
| add ax, 158 | F400, 0154 |
| mov sum, ax | 0700,0100 |
| mov ax, 0 | 0700,010F |

LAB 5:

ACTIVITY 5.1:

Define and Display 8-bit Numbers. Write a program to do the following , Use the DB directive to define the following list of numbers and name it array. 31h, 32h, 33h, 34h ,  Use the int 21h to display the array. Execute and display outcome here?

Answer:

org 100h

.data

array db "10h, 20h, 30h, 40h", '$'

.code

main proc

mov ax, @data

mov ds, ax

mov dx, offset array

mov ah, 9h

int 21h

mov ax, 4c00h

int 21h

main endp

end main

Ex 5.1:

For each add instruction in this exercise, assume that AX contains the given contents before the instruction is executed. Give the contents of AX as well as the values of the CF, OF, SF, PF, AF and ZF after the instruction is executed. All numbers are in hex. (Hint: add ax, 45 adds 45 to the contents of register ax and stores the result back in ax) ?

| Contents of AX (Before) | Instruction | Contents of AX (After) | CF | OF | SF | PF | AF | ZF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0045 | add ax, 45 | 008A | 0 | 0 | 1 | 1 | 0 | 0 |
| FF45 | add ax, 45 | FF8A | 0 | 0 | 1 | 1 | 0 | 0 |
| 0045 | add ax, -45 | 0000 | 1 | 0 | 0 | 1 | 1 | 1 |
| FF45 | add ax, -45 | FF00 | 1 | 0 | 1 | 1 | 1 | 0 |
| FFFF | add ax, 1 | 0000 | 1 | 0 | 0 | 1 | 1 | 1 |

Answer:

CF (Carry): Set if there is an unsigned overflow (like adding 1 to FFFF).

ZF (Zero): Set if the result is exactly zero.

SF (Sign): Set if the MSB (Most Significant Bit) is 1 (negative result).

PF (Parity): Set if the lower 8 bits of the result have an even number of 1s.

Ex 5.2:

Calculate the range of signed and unsigned numbers that db and dw directives can specify?

Answer:

The range depends on whether the numbers are interpreted as signed (Two's Complement) or unsigned.

DB (Define Byte = 8 bits):

Unsigned: 0 to 255 (2^8 - 1)

Signed: -128 to +127

DW (Define Word = 16 bits):

Unsigned: 0 to 65,535 (2^16 - 1)

Signed: -32,768 to +32,767

Ex 5.3:

Write a program that will display uppercase letters (A-Z), using loop on new line.

Answer:

| Directive | Initial Value (Hex/ASCII) |
| --- | --- |
| byte1 db 10110111b | B7h |
| byte2 db 35q | 1Dh (35 octal = 29 decimal) |
| byte3 db 0E8h | E8h |
| byte4 db 150 | 96h |
| byte1 db -91 | A5h (Two's complement of 91) |
| byte1 db 'K' | 4Bh (ASCII for 'K') |
| byte1 db 'k' | 6Bh (ASCII for 'k') |
| byte1 db "Ali's book" | 41 6C 69 27 73 20 62 6F 6F 6B |
| byte1 db 5 DUP("< >") | 3C 20 3E (repeated 5 times) |
| byte1 dw 1000000 | Error (Value too large for 16-bit DW) |

LAB 6:

ACTIVITY 6.1: WORD MULTIPLICATION?

Answer:

include 'emu8086.inc'

org 100h

.data

num1 db 2

num2 db 2

.code

main proc

mov al,2

mov bl,2

mul bl

call print_num

;add ax,30h

;mov dx,ax

;mov ah,2h

;int 21h

;mov ax,4c00h

;int 21h

main endp

define_print_num

define_print_num_uns

end main

ACTIVITY 6.2: USE OF CBW?

Answer:

include 'emu8086.inc'

org 100h

.code

main proc

mov al,4

cbw

mov bl,2

div bl

call print_num

main endp

define_print_num

define_print_num_uns

end main

include 'emu8086.inc'

org 100h

.code

main proc

mov ax,-4

cwd

mov bx,2

idiv bx

call print_num

main endp

define_print_num

define_print_num_uns

end main

ACTIVITY 6.3: SIMPLE PROGRAM FOR CELSIUS – FAHRENHEIT CONVERSION The working formula is f = (9/5) *c + 32?

Answer:

include 'emu8086.inc'

org 100h

.data

c db 20

.code

main proc

mov al,-9

imul c

mov bl,5

idiv bl

cbw

add ax,32

call print_num

main endp

define_print_num

define_print_num_uns

end main

Exercise 6.1: Write a program to use the SCAN_NUM to input prompt for values for three variables x, y and z and the PRINT_NUM

to display an appropriate label and value of the expression x – y + 2z – 1. Write code and outcome here?

Answer:

include 'emu8086.inc'

org 100h

.data

c dw 2

s db 0dh,0ah,'$'

.code

main proc

print 'Enter value of x: '

call scan_num

mov ax,@data

mov ds,ax

mov dx,offset s

mov ah,9h

int 21h

mov ax,cx

print 'Enter value of y: '

call scan_num

sub ax,cx

mov bx,ax

mov ax,@data

mov ds,ax

mov dx,offset s

mov ah,9h

int 21h

print 'Enter value of z: '

call scan_num

mov ax,cx

mul c

add bx,ax

sub bx,1

mov ax,@data

mov ds,ax

mov dx,offset s

mov ah,9h

int 21h

mov ax,bx

print 'The answer is: '

call print_num

main endp

define_scan_num

define_print_num

define_print_num_uns

end main

Exercise 6.2:

Write an assembly language program that prompts for and inputs the length, width, and height of a box and calculates and displays its surface area. surface area = 2 * (length * width + length * height + width * height) Write code and outcome here?

Answer:

;surface area = 2 * (length * width + length * height + width * height)

include 'emu8086.inc'

org 100h

.data

o db 0dh,0ah,'$'

m dw 2

l dw 0

w dw 0

h dw 0

.code

main proc

print 'Enter length: '

call scan_num

mov ax,@data

mov ds,ax

mov dx,offset o

mov ah,9h

int 21h

mov l,cx

print 'Enter width: '

call scan_num

mov ax,@data

mov ds,ax

mov dx,offset o

mov ah,9h

int 21h

mov w,cx

print 'Enter height: '

call scan_num

mov ax,@data

mov ds,ax

mov dx,offset o

mov ah,9h

int 21h

mov h,cx

mov ax,l

mul w

mov bx,ax

mov ax,l

mul h

mov cx,ax

mov ax,w

mul h

add bx,cx

add bx,ax

mov ax,bx

mul m

print 'surface area = '

call print_num

main endp

define_scan_num

define_print_num

define_print_num_uns

end main

# LAB 7

ACTIVITY 7.1:

Solution:

include 'emu8086.inc'

org 100h

.code

main proc

mov al,2

mov bl,3

neg bl

add al,bl

cbw

call print_num

main endp

define_print_num

define_print_num_uns

end main

include 'emu8086.inc'

org 100h

.code

main proc

mov ax,2

mov bx,3

neg bx

add ax,bx

call print_num

main endp

define_print_num

define_print_num_uns

end main

Exercise 7.1:

Solution:

org 100h

.code

main proc

mov al, 00001111b

and al, 00111011b

nop

mov al, 6Dh

and al, 4Ah

nop

mov al, 00001111b

or  al, 61h

nop

mov al, 94h

xor al, 37h

nop

main endp

end main

OUTPUT:

For these operations, we compare bits: AND requires both to be 1, OR requires at least one to be 1, and XOR requires exactly one to be 1.

- a. 00001111 AND 00111011 = 00001011b ( 0Bh )

- b. 6Dh (01101101) AND 4Ah (01001010) = 01001000b ( 48h )

- c. 00001111 OR 61h (01100001) = 01101111b ( 6Fh )

- d. 94h (10010100) XOR 37h (00110111) = 10100011b ( A3h )

Exercise 7.2:

Solution:

org 100h

.code

main proc

mov al, 7Ah

not al

nop

mov al, 3Dh

and al, 74h

nop

mov al, 9Bh

or al, 35h

nop

mov al, 72h

xor al, 0dch

nop

main endp

end main

OUTPUT:

- a. NOT 7Ah: NOT flips all bits. 7A (0111 1010) becomes 1000 0101 = 85h

- b. 3Dh (0011 1101) AND 74h (0111 0100) = 34h

- c. 9Bh (1001 1011) OR 35h (0011 0101) = BFh

- d. 72h (0111 0010) XOR DCh (1101 1100) = AEh

Exercise 7.3:

Solution:

org 100h

.code

main proc

mov al,00001111b

test al,00000010b

nop

mov al,00000110b

cmp al,00000101b

nop

mov al,00000101b

cmp al,00000111b

nop

main endp

end main

Write a single instruction that clears  the high 8 bits of AX and does not change the low 8 bits?

Exercise 7.4:

Solution:

Mov  ax , 1234h

AND   AX , 00FFH

Write a single instruction that sets  the high 8 bits of AX and does not change the low 8 bits?

Exercise 7.5:

Solution:

Mov  ax , 1234h

OR     AX , FF00H

| Operation | Example MOV | Instruction | Result |
| --- | --- | --- | --- |
| Clear High 8 Bits | MOV AX , 1234H | AND AX , 00FFH | AX = 0034H |
| Clear Low 8 Bits | MOV AX , 1234H | AND AX , FF00H | AX = 1200H |
| Set High 8 Bits | MOV AX , 1234H | OR AX , FF00H | AX = FF34H |
| Set Low 8 Bits | MOV AX, 1234H | OR AX , 00FFH | AX = 12FFH |

Only For Understanding:-

✨: call print_num = ax

✨: call scan_num = cx

✨: mov ah,1h = al

✨: call GET_STRING ,call  PRINT_STRING   = dx

✨: when ever we do mul ax or mul variable then it do multiplication with ax and stores result in ax

we imul for negative no *

✨: when ever we do div ax or div variable then it do division with ax and stores result in ax

Remember cbw used when using byte register to convert byte to word

Remember cwd used when using word register to convert word to double

we idiv for negative no division